Scope of Work

PROJECT NAME:		Taylors Ferry

PROJECT LOCATION:		2401 SW Taylors Ferry Rd, Portland, OR, 97219

PROJECT DESCRIPTION:		17 Homes

SCOPE OF WORK:		             Flooring

DATE OF WORK:		Summer 2026

BID DUE DATE:		03/27/2026 4 pm

Scope of Work:

2 Mobilizations per building

1 - Site Walk

2 - LVP Install

Send Habitat submittal information within 5 days of contract execution

Scope

- Materials

  - Contractor to supply all materials, including but not limited to LVP flooring, transitions, caulking, and ram board.

  - Flooring product will be MSI Everlife Ashton interlocking LVP or approved equal. Homeowners to choose from 3 colors.

  - Full stair tread product with square nose to be used at stairs.

  - Material prices to be guaranteed through the duration of the project.

- Installation

  - Contractor responsible for walking the building prior to install and prepping all wood sub-floors as needed for flooring installation.

  - Unit will be left broom clean by previous trade. Include vacuuming before install.

  - Install LVP flooring per manufacturer specifications.

  - Note: Crawlspace floor access hatches- no detail shown, contractor to provide mockup

  - Install caulk at LVP / tub expansion gap

  - Install ram board floor protection paper over LVP the same day that each level of the home is completed.

  - Leave 1 extra box of LVP in first floor closet for homeowner